LEDTimerControl help document 

I. Main interface of software description: as shown in figure 1-1 .


Figure 1-1

1.Menu bar: it contains package menu ,mode menu ,control menu ,language menu and help menu .


(1) Package menu :it is specialized for  the mode package controlling /processing menu ;

       New : means to create a new mode package ,50 modes are included . 

       Open : is to unfold an existing mode package .

       Save : to save the current mode package and all mode file ,and do remember to preserve

             mode package after setup value.
       Exit : to close the application program.
(2)  Mode menu : it is a menu for mode control , it will be available when there is a mode bag ;

        Insert :you can use it to create a new mode file which includes max 50 time points , and it 

              can be download to the controller when it has at least two time points .

Load : to load an existing mode, and mode name should be not the same;

Edit : Used to open the figure 1-2 . It can be opened when mode is selected . Also it can

     be turned on via double-click edit display frame .

Export : to save the current editing mode to the file , and the data of time point edited

       won't be lost when software is closed or reboot .

Delete :to delete mode file , and you can remove it out of the list alone ,once mode file is

       deleted , it can't be opened again.
MoveUp: used to change the mode download sequence, and pattern moved up

MoveDown : mode is moving down ;

Rename : you can rename the mode ;

Empty : Remove all patterns of mode list in the current mode package .

(3)  Control menu :it is used to manage and setup the controller . Only when controller 

                     inserted , the option is available.
Download: In sequence to download all patterns of mode list in mode package to the

          controller, if time point number is less than 2, then skip the download ;

OnlinePlay :Quick preview mode setting effect, when previewing , most of the operation 

           will be unable, and it will be restored after previewed .

SynTime :The time of controller can be set into computer time via synchronous clock .

ClearAll : used to delete all modes saved in the controller .

(4)  Language menu : the option for language switch .

(5)  Help menu : to view the help and check the information .

2. Standard toolbar :options are as followed : new package , open package , save package , fast 

   sowing, download and synchronization and so on .

3. Mode tool sample :functional key are such as nouveau mode , delete , open , save and move 

   mode .

4. Mode list :Display all mode in the current mode package , double-click the mode name to

   change name, then push enter or other key to save rename , and click ESC to cancel.
5. Device type : show the current setting mode package to be suitable for which device type .

6. Edit display frame : the editing schedule, step number and the channel value can be displayed .

7. Cycle mark : fast sowing

8. The speed of play ,1 time speed is not accelerating ,2 times speed of play time is half of the

  actual time, 4 times speed of play time is 1/4 of the actual time , 8 times speed of play time is 

  1/8 of the actual time , 16 times speed of play time is 1/16 of the actual time ;

9. Package name of the current using mode , and the package is used for storage and 

  management mode. Make sure the package name when open or new construction.
II. Interface description of step edit box : as shown in figure 1-2
[image: image1.bmp]
Figure 1-2

1. Step number : the front number means the current setting step , and the behind number is total

   steps , the maximum total steps are fifty .

2. Time editor : to set up hour and minute separately according to 24-hour time system .

3. Channel value setting : the channel value is measured via percentage , and fade means the

  brightness of this channel is gradual changing into the next step in the same channel .

  Prev: to check last step ;

  Next: to see next step ;

  Close: to close setting ;

  Add: when adding a step , time will automatically add a minute , but the time of adding step can't 

      be the same as the other step number .

  Delete: to delete the editing step;

III. Direction for use 

Step 1: New established mode package , to click the first button of "common" toolbar or select "Package" - > "New", and open the "Package Options", as shown in figure 1-3, "PackagePath" are mode Package files and mode storage path, "PackageName" are mode package and mode file name; "DeviceType" is equipment pattern , "ChannelNum" is channel number equipment needed . After setup is completed, click "OK";

[image: image2.png]Package Options

Packagebath; PLed [ |

Packageliame: [NenPackage

DeviceType :  [1caz0

Channelium





Figure 1-3

Step 2 : creating a new mode : click the first button "Mode" on the toolbar or select "Mode" - > "Insert", the new mode name and mode file name will be defaulted "Mode_" at the beginning , as shown in figure 1-1.

Step 3 : editing mode: firstly click the last button on "mode" toolbar or select "Mode" - > "edit" option, open the "step edit box", as shown in figure 1-2; And click the "Add" button to add a time point , then edit channel value and modify the procedure time , at last step number will be automatic changed according to the modified time .

Step 4: check step edit effect: when the step edit meets requirement, choose "Control" - > "play" or button Alt + p key, or click the "common" play button on the toolbar, to see the editing effect. If you edit appropriately, then download to the controller, or continue to modify.

Step 5: Download mode: when step edit is correct, choose the "Control" - > "Download" option, or click the "Download" button on "common" toolbar, or click the ALT + L key at the same time to get download mode.

Step 6: Save mode, select "Package" - > "Save" or click  "Save" button on the "common" on the toolbar , or press Ctrl + S key at the same time, Save all model and mode package , and then exit the software.

































